DIY SMARTWATCH

Raspberry Pi Pico 2W  +  Rust Embedded

Technical Documentation

A fully functional smartwatch built on a breadboard, featuring:

• Real-time clock display (DS3231)

• Heart rate monitoring (MAX30102)

• Motion detection (MPU6050)

• 240×240 round TFT display (GC9A01)

• LiPo battery with USB-C charging

• Written in Rust using Embassy framework

Version 1.0  |  May 2026  |  Bucharest, Romania

# 1. Project Overview

This project is a DIY smartwatch built on a breadboard using the Raspberry Pi Pico 2W microcontroller, programmed entirely in Rust. The goal was to create a working smartwatch prototype while learning embedded systems programming with the Rust language and the Embassy async framework.

The smartwatch communicates with its peripherals using two standard protocols: SPI (Serial Peripheral Interface) for the high-speed display, and I2C (Inter-Integrated Circuit) for all sensors. The Pico 2W acts as the bus master for all communications.

The project was built step-by-step: hardware assembly on breadboard → MicroPython testing to verify connections → Rust firmware development → final integration.

## Key Features

- Real-time clock display read from DS3231 RTC (backed by CR2032 battery)

- Heart rate and SpO2 monitoring via MAX30102 optical sensor

- Step counting and motion detection via MPU6050 6-axis IMU

- 1.28" circular GC9A01 TFT display (240×240 pixels, RGB565)

- LiPo battery with TP4056 USB-C charging circuit

- Two physical push buttons for user interaction

- Built-in WiFi and Bluetooth (Raspberry Pi Pico 2W)

- Firmware written in Rust — memory safe, zero-cost abstractions

## Programming Language & Framework

The firmware is written in Rust (no_std environment) using the Embassy embedded async framework. Rust provides memory safety without a garbage collector, making it ideal for embedded systems.

Target microcontroller architecture:

thumbv8m.main-none-eabihf  →  ARM Cortex-M33 (RP2350 on Pico 2W)

The Rust ecosystem used:

embassy-rp      = "0.1"   # RP2350 Hardware Abstraction Layer

embassy-executor = "0.5"  # Async task executor

embassy-time    = "0.3"   # Timing primitives

embedded-graphics = "0.8" # 2D graphics rendering

ds323x          = "0.5"   # DS3231 RTC driver

cortex-m-rt     = "0.7"   # Cortex-M runtime

# 2. Components List & Costs

All components were sourced online from AliExpress, Romanian electronics shops, and local supermarkets. Prices are approximate and in Romanian Lei (RON).

| Component | Model | Role | Source | Price (RON) | Protocol |
| --- | --- | --- | --- | --- | --- |
| Raspberry Pi Pico 2W | RP2350 | Main MCU + WiFi/BT | AliExpress | ~55 RON | USB / SPI / I2C |
| Round TFT Display | GC9A01 1.28" | Smartwatch screen | AliExpress | ~25 RON | SPI |
| RTC Module | DS3231 ZS-042 | Real-time clock | AliExpress | ~15 RON | I2C (0x68) |
| Heart Rate Sensor | MAX30102 | Pulse & SpO2 | AliExpress | ~20 RON | I2C (0x57) |
| Accel. / Gyroscope | MPU6050 HW-123 | Motion / pedometer | AliExpress | ~12 RON | I2C (0x69) |
| LiPo Charger | TP4056 USB-C | Battery charging | AliExpress | ~8 RON | — |
| LiPo Battery | 3.7V 250mAh | Power supply | akyga.com | ~20 RON | — |
| Tactile Buttons x2 | 6×6mm THT | User input | Local shop | ~2 RON | GPIO |
| Breadboards x2 | 400 + 830 pt. | Component mounting | AliExpress | ~25 RON | — |
| Jumper Wire Kit | M-M / M-F | Connections | AliExpress | ~10 RON | — |
| Resistor Kit | 600pcs 1/4W | Pull-ups / limits | AliExpress | ~15 RON | — |
| CR2032 Battery x4 | Duracell 3V | DS3231 backup | Supermarket | ~20 RON | — |
| TOTAL ESTIMATED |  |  |  | ~227 RON |  |

# 3. Hardware Connections

The system uses two communication buses: SPI1 for the high-speed display, and I2C for all sensors (shared bus). Power (3.3V) is provided by the Pico 2W's onboard LDO regulator, fed by the LiPo battery via TP4056.

## 3.1  Display — GC9A01  (SPI1)

The GC9A01 is a 1.28" round TFT with 240×240 RGB565 resolution. It uses SPI1 at up to 32 MHz. RST is held high after initialization.

| Display Pin | Pico 2W Pin | GP Number |
| --- | --- | --- |
| VCC | 3V3 OUT | Pin 36 |
| GND | GND | Pin 33 |
| SCL (SCK) | GP10 | Pin 14 |
| SDA (MOSI) | GP11 | Pin 15 |
| DC | GP8 | Pin 11 |
| CS | GP9 | Pin 12 |
| RST | GP12 | Pin 16 |

## 3.2  DS3231 Real-Time Clock  (I2C, 0x68)

The DS3231 is an ultra-precise RTC with integrated temperature-compensated crystal oscillator. A CR2032 coin cell keeps time when the main power is removed. I2C address: 0x68.

| DS3231 Pin | Pico 2W Pin | GP Number |
| --- | --- | --- |
| VCC | 3V3 OUT | Pin 36 |
| GND | GND | Pin 33 |
| SDA | GP26 | Pin 31 |
| SCL | GP27 | Pin 32 |

## 3.3  MAX30102 Heart Rate Sensor  (I2C, 0x57)

The MAX30102 uses a red LED (660 nm) and an infrared LED (880 nm) to detect photoplethysmographic (PPG) signals for heart rate and SpO2 measurement. I2C address: 0x57.

| MAX30102 Pin | Pico 2W Pin | GP Number |
| --- | --- | --- |
| VIN | 3V3 OUT | Pin 36 |
| GND | GND | Pin 33 |
| SDA | GP26 | Pin 31 |
| SCL | GP27 | Pin 32 |

## 3.4  MPU6050 Accelerometer & Gyroscope  (I2C, 0x69)

The MPU6050 provides 3-axis acceleration ±2g–±16g and 3-axis gyroscope ±250–±2000 °/s. Its default I2C address 0x68 conflicts with DS3231, so the AD0 pin is pulled HIGH to change it to 0x69.

| MPU6050 Pin | Pico 2W Pin | Notes |
| --- | --- | --- |
| VCC | 3V3 OUT | 3.3V supply |
| GND | GND | Common ground |
| SDA | GP26 | Shared I2C bus |
| SCL | GP27 | Shared I2C bus |
| AD0 | 3V3 | Address → 0x69 |

## 3.5  Power Supply — TP4056 + LiPo 3.7V

The TP4056 module charges the LiPo battery at up to 1A via USB-C. Its output feeds the Pico 2W's VSYS pin. The Pico 2W has an internal RT6150 buck-boost converter that produces a stable 3.3V for all peripherals.

| TP4056 Pin | Connected To | Notes |
| --- | --- | --- |
| IN+ / IN- | USB-C port | 5V charging input |
| BAT+ / BAT- | LiPo 3.7V | JST connector |
| OUT+ | VSYS (Pico 2W) | Battery voltage out |
| OUT- | GND | Common ground |

# 4. Software Architecture

The firmware is structured as a bare-metal Rust application with no operating system. The Embassy framework provides cooperative async/await concurrency suitable for embedded systems.

## 4.1  Project Structure

smartwatch/

├── src/

│   └── main.rs        # Main firmware — init + main loop

├── Cargo.toml         # Dependencies and build config

├── memory.x           # Linker script for RP2350 memory layout

├── .cargo/

│   └── config.toml    # Rust target and runner configuration

└── README.md          # Project documentation

## 4.2  Memory Layout (RP2350)

MEMORY {

FLASH : ORIGIN = 0x10000000, LENGTH = 2048K

RAM   : ORIGIN = 0x20000000, LENGTH = 520K

}

## 4.3  Custom GC9A01 Display Driver

No compatible Rust crate existed for the GC9A01 with Embassy 0.1 (embedded-hal version conflict), so a complete custom SPI driver was implemented. It implements the embedded-graphics DrawTarget trait for text and shape rendering.

Driver initialization sequence:

- Toggle RST pin LOW → HIGH with delays

- Send 40+ initialization commands via SPI (manufacturer sequence)

- Set pixel format: RGB565 (16-bit, 0x3A = 0x05)

- Send Sleep Out (0x11) → wait 120ms

- Send Display On (0x29) → wait 20ms

Pixel rendering uses the RAMWR command (0x2C) with column/row address set commands (0x2A, 0x2B) before each pixel write.

## 4.4  Main Application Loop

The main loop runs at approximately 500ms intervals:

- Read current time from DS3231 over I2C

- Clear the display (fill with black)

- Render clock time as white text at display center

- Render heart rate (BPM) as text below clock

- Check button GPIO states for user interaction

- Delay 500ms via cortex_m::asm::delay()

## 4.5  Build & Flash Instructions

Prerequisites: Rust toolchain, elf2uf2-rs tool.

1. Add the ARM Cortex-M33 target:

rustup target add thumbv8m.main-none-eabihf

2. Build the release binary:

cargo build --release

3. Enter BOOTSEL mode on Pico 2W:

Hold the BOOTSEL button, plug in USB, release the button. The RP2350 drive appears.

4. Flash the firmware:

elf2uf2-rs -d target/thumbv8m.main-none-eabihf/release/smartwatch

# 5. Challenges & Solutions

Building this project required solving several non-trivial technical problems. Here is a summary:

| Challenge | Root Cause | Solution |
| --- | --- | --- |
| MPU register bug in rp2040-hal | New Rust compiler changed cortex-m MPU register layout (RASR removed) | Replaced install_stack_guard() with empty Ok(()) stub |
| GC9A01 crate incompatible | gc9a01 v0.2 requires embedded-hal 0.2, Embassy needs 1.0 — version conflict | Wrote custom SPI driver implementing embedded-graphics DrawTarget |
| Display not responding | Wrong GPIO pins connected (breadboard wiring errors) | Used MicroPython to scan all pin combos and identify correct GPIOs |
| I2C address conflict | DS3231 and MPU6050 both default to I2C address 0x68 | Connected MPU6050 AD0 pin to 3V3 → address changes to 0x69 |
| embassy-time linker error | Missing _embassy_time_schedule_wake symbol when using async Timer | Replaced async Timer with blocking cortex_m::asm::delay() |
| Defective display module | GC9A01 module was damaged — no response even to RST signal | Ordered replacement; firmware is ready and tested to compile correctly |
| TP4056 has no breadboard pins | TP4056 uses SMD pads, not through-hole pins | Soldered wires directly to SMD pads using soldering iron |

# 6. Future Improvements

- NTP time synchronization via built-in WiFi (Pico 2W)

- Bluetooth Low Energy (BLE) notifications from smartphone

- Proper heart rate algorithm with peak detection on MAX30102 data

- Step counter implementation using MPU6050 accelerometer data

- 3D-printed wristband case designed in FreeCAD

- Custom PCB designed in KiCad to replace the breadboard

- Menu system navigated via the two push buttons

- Power-saving deep sleep mode between display updates

- Over-the-air (OTA) firmware updates via WiFi

- Real-time data logging to MicroSD card

# 7. References

| Resource | Description / URL |
| --- | --- |
| Embedded Rust 101 | https://embedded-rust-101.wyliodrin.com — Course used for reference |
| Embassy Framework | https://embassy.dev — Async embedded Rust framework |
| Pico 2W Datasheet | https://datasheets.raspberrypi.com/pico/pico-2-datasheet.pdf |
| embedded-graphics | https://docs.rs/embedded-graphics — 2D graphics for embedded systems |
| ds323x crate | https://docs.rs/ds323x — DS3231 Rust driver |
| GC9A01 Datasheet | Galaxycore GC9A01 LCD controller datasheet |
| MAX30102 Datasheet | Maxim Integrated MAX30102 High-Sensitivity Pulse Oximeter |
| MPU6050 Datasheet | InvenSense MPU-6050 Product Specification Revision 3.4 |

DIY Smartwatch — Raspberry Pi Pico 2W + Rust  |  Technical Documentation v1.0  |  May 2026